RESUME

# A Physics-Informed Closed-Loop Digital Twin Framework for Personalized rTMS in Treatment-Resistant Depression Using Multimodal EEG–ERP–ECG Biomarkers

## Abstract

Background: Repetitive transcranial magnetic stimulation (rTMS) is an established non-invasive neuromodulation therapy for treatment-resistant depression (TRD). However, conventional rTMS protocols are predominantly open-loop and rely on fixed stimulation parameters, limiting treatment personalization and potentially reducing clinical efficacy.

Objective: This study proposes a novel closed-loop adaptive framework that integrates electroencephalography (EEG), event-related potentials (ERP), and electrocardiography (ECG) biomarkers with a physics-informed artificial intelligence architecture to optimize rTMS therapy in real time.

Methods: The proposed framework combines four complementary components: (i) multimodal physiological acquisition (EEG, ERP, and ECG), (ii) Bayesian estimation of the latent brain state, (iii) a patient-specific digital twin based on stochastic Wilson–Cowan neural dynamics coupled with a Physics-Informed Long Short-Term Memory (PINN-LSTM) network, and (iv) a Model Predictive Controller (MPC) that continuously adapts stimulation parameters according to the estimated neurophysiological state. The learning process is constrained by neurophysiological equations, ensuring consistency between artificial intelligence predictions and cortical dynamics.

Expected Results: The proposed framework is expected to improve prediction accuracy, increase robustness to physiological variability, reduce unnecessary stimulation energy, and enable individualized rTMS optimization through continuous multimodal feedback.

Conclusion: The proposed framework introduces a unified methodology combining computational neuroscience, stochastic modeling, physics-informed artificial intelligence, Bayesian inference, digital twins, and optimal control. By integrating neurophysiological knowledge with adaptive machine learning, the framework represents a promising step toward personalized closed-loop neuromodulation for treatment-resistant depression.

Keywords: Treatment-resistant depression; repetitive transcranial magnetic stimulation; EEG; ERP; ECG; Physics-Informed Neural Networks; Digital Twin; Bayesian Estimation; Model Predictive Control; Closed-Loop Neuromodulation.

# 1. Introduction

## 1.1 Background

Treatment-resistant depression (TRD) remains one of the most challenging psychiatric disorders, affecting a substantial proportion of patients who fail to respond adequately to conventional pharmacological interventions. Repetitive transcranial magnetic stimulation (rTMS) has become an effective non-invasive neuromodulation technique capable of modulating cortical excitability and promoting neuroplasticity. Nevertheless, current clinical practice generally relies on predefined stimulation protocols that do not account for the continuously changing physiological state of the patient's brain.

Recent advances in artificial intelligence have demonstrated encouraging results for predicting therapeutic response using electroencephalography (EEG). Despite these achievements, most existing methods are based on black-box models trained solely on EEG recordings and operate in an open-loop manner, providing predictions without dynamically adapting stimulation parameters.

Furthermore, depression involves complex interactions between cortical activity, cognitive processing, and autonomic nervous system regulation. Consequently, relying on a single physiological modality may overlook complementary biomarkers that could improve prediction accuracy and treatment personalization.

In this work, we propose a multimodal closed-loop framework integrating EEG, event-related potentials (ERP), and electrocardiography (ECG). Unlike previous studies, the proposed system combines stochastic neurophysiological modeling, Bayesian state estimation, physics-informed deep learning, and optimal control within a unified patient-specific digital twin capable of continuously adapting rTMS stimulation according to the estimated brain state.

2. Related Work

## 2.1 Artificial Intelligence for rTMS Response Prediction

Recent advances in artificial intelligence have substantially improved the prediction of treatment response in major depressive disorder using electroencephalography (EEG). Deep learning architectures, including Long Short-Term Memory (LSTM), Convolutional Neural Networks (CNN), Transformer models, and hybrid approaches, have demonstrated promising predictive performance. However, these models are generally trained as black-box predictors and rarely incorporate neurophysiological constraints, limiting their interpretability and clinical adoption. Recent systematic reviews emphasize the need for more explainable and physiologically grounded AI methods.

## 2.2 EEG Biomarkers for rTMS

EEG remains the principal modality for monitoring cortical activity during rTMS. Several studies have identified biomarkers such as frontal alpha asymmetry, theta-band connectivity, beta oscillations, and the N100 component as potential predictors of therapeutic response. Nevertheless, the reported biomarkers are heterogeneous across studies, and no consensus has yet emerged regarding a robust biomarker set suitable for individualized treatment planning.

## 2.3 Closed-Loop rTMS

Closed-loop neuromodulation has recently emerged as a promising strategy to personalize brain stimulation. Most existing systems use EEG alone to trigger stimulation according to oscillatory phase or employ reinforcement learning to optimize stimulation timing. Although these studies demonstrate the feasibility of adaptive EEG-guided TMS, they generally focus on a single physiological modality and do not integrate patient-specific physiological models or digital twins.

## 2.4 Multimodal Physiological Information

Depression is not only associated with cortical dysfunction but also with alterations in cognitive processing and autonomic nervous system regulation. Consequently, integrating EEG with event-related potentials (ERP) and electrocardiography (ECG) may provide complementary information about cortical dynamics, cognitive function, and autonomic regulation. Recent meta-analyses have shown that multimodal AI generally outperforms unimodal approaches for depression-related tasks, highlighting the potential value of physiological data fusion.

## 2.5 Current Research Gaps

Despite significant progress, several important limitations remain:

- Most studies rely exclusively on EEG.

- ERP and ECG are rarely integrated into adaptive rTMS systems.

- Existing deep learning models often lack physiological interpretability.

- Few studies estimate the latent brain state using Bayesian inference.

- Closed-loop control strategies remain uncommon in clinical rTMS.

- To the best of our knowledge, no previous study has jointly integrated EEG, ERP, ECG, stochastic neural modeling, Physics-Informed Neural Networks, Bayesian state estimation, Model Predictive Control, and a patient-specific digital twin within a unified closed-loop framework for treatment-resistant depression.

## 2.6 Contributions of the Present Study

To address these limitations, the present work proposes a unified computational framework that combines:

- Multimodal physiological biomarkers (EEG, ERP, and ECG).

- A stochastic Wilson–Cowan neural population model.

- Bayesian estimation of the latent brain state.

- A Physics-Informed LSTM architecture constrained by neurophysiological equations.

- A patient-specific digital twin capable of continuously updating the estimated cortical state.

- A Model Predictive Controller that optimizes rTMS parameters in real time through closed-loop feedback.

This integrated framework aims to improve prediction accuracy, physiological interpretability, treatment personalization, and adaptive neuromodulation compared with existing approaches.

# 3. Proposed Closed-Loop Physics-Informed Digital Twin Framework

## 3.1 Motivation

Les systèmes actuels de rTMS présentent plusieurs limites :

- stimulation en boucle ouverte ;

- utilisation d'un seul biomarqueur (EEG) ;

- absence d'estimation de l'état cérébral latent ;

- absence d'un modèle physiologique intégré ;

- absence de personnalisation dynamique.

Pour surmonter ces limitations, nous proposons une plateforme adaptative en boucle fermée reposant sur trois niveaux d'information physiologique (EEG, ERP et ECG), un modèle neurophysiologique, une IA contrainte par la physique et un contrôleur prédictif.

# Figure 1 : Architecture générale

EEG        ERP        ECG
                  │          │          │
                  └──────┬───┴──────────┘
                         │
              Signal preprocessing
                         │
                         ▼
              Feature Extraction Layer
                         │
                         ▼
           Bayesian State Estimation (EKF)
                         │
                         ▼
           Patient-Specific Digital Twin
            (Wilson–Cowan + PINN + LSTM)
                         │
                         ▼
           Therapeutic Response Index (TRI)
                         │
                         ▼
             Model Predictive Controller
                         │
                         ▼
           Adaptive rTMS Parameters
     (Intensity, Frequency, Pulse Number)
                         │
                         ▼
                   Patient Brain
                         ▲
                         │
                 Closed-loop Feedback

# Description scientifique des blocs

## Bloc 1 : Acquisition multimodale

EEG(t)

ERP(t)

ECG(t)​​

Les observations sont définies par :

y(t)=​                           +v(t)

où :

- EEG(t): activité électrique corticale ;

- ERP(t): réponse cognitive évoquée ;

- ECG(t): activité autonome ;

- v(t): bruit de mesure.

## Bloc 2 : Estimation de l'état cérébral

L'état latent est défini par :

x(t)=[E(t),I(t),A(t),C(t)]T

avec :

- E(t) : activité excitatrice ;

- I(t) : activité inhibitrice ;

- A(t) : état autonome dérivé de l'ECG ;

- C(t) : état cognitif dérivé de l'ERP.

Cette définition est une contribution intéressante : elle intègre explicitement des composantes corticales, autonomes et cognitives dans un même état.

## Bloc 3 : Dynamique du cerveau

Nous décrivons l'évolution de l'état par :

dx(t)=f(x(t),u(t))dt+ΣdW(t)

où :

- u(t) représente les paramètres de la rTMS ;

- Σ modélise la variabilité inter- et intra-patient.

## Bloc 4 : Jumeau numérique

Le jumeau numérique estime en continu :

x̂(t)= FPINN−LSTM(y(t))

Il apprend la dynamique du patient tout en respectant les contraintes neurophysiologiques imposées par le modèle de Wilson–Cowan.

## Bloc 5 : Indice thérapeutique

Nous proposons un nouvel indicateur :

TRI(t)=σ(W  x̂ (t) + b)

avec :

- TRI∈[0,1] ;

- TRI≈1TRI : réponse thérapeutique favorable ;

- TRI≈0TRI : faible probabilité de réponse.

Remarque importante : cet indice est une proposition méthodologique. Il devra être validé expérimentalement et ne doit pas être présenté comme un biomarqueur établi.

## Bloc 6 : Contrôle adaptatif

Le contrôleur calcule :

u∗=arg minJ

avec :

J= 0k  (xk−xref)T  Q(xk−xref)+ ukT Ruk

Le but est de rapprocher l'état cérébral de l'état cible tout en limitant l'intensité de la stimulation.

# Pourquoi cette architecture est-elle intéressante ?

Par rapport à la littérature actuelle, elle réunit dans un même cadre :

- la fusion multimodale EEG + ERP + ECG ;

- un modèle neurophysiologique ;

- une estimation bayésienne de l'état latent ;

- un jumeau numérique personnalisé ;

- un contrôle prédictif en boucle fermée.

Cette intégration est potentiellement une contribution originale, à condition qu'elle soit accompagnée d'une validation rigoureuse et d'une comparaison honnête avec les travaux existants.

Section 4 : Mathematical Model

C'est cette section qui fera la différence avec les articles classiques utilisant uniquement des réseaux de neurones.

Je vous propose toutefois une amélioration importante.

### Au lieu de simplement assembler plusieurs modèles, nous allons définir un modèle mathématique unifié.

L'idée est que toutes les équations décrivent le même système :

Patient → EEG-ERP-ECG → Eˊtat latent → Digital Twin → Optimisation → rTMS

Chaque bloc sera relié au suivant par une équation, ce qui donnera un cadre mathématique cohérent.

On recommande aussi de ne pas multiplier inutilement les équations. Pour une revue de haut niveau, un petit nombre d'équations bien justifiées est souvent plus convaincant qu'un grand nombre d'équations disparates.

À partir de l'architecture que nous avons définie, les équations principales seront :

- Dynamique cérébrale stochastique (évolution de l'état latent).

- Modèle d'observation multimodale (EEG, ERP, ECG).

- Wilson–Cowan (physiologie des populations neuronales).

- Estimation bayésienne (reconstruction de l'état latent).

- PINN-LSTM (prédiction contrainte par la physiologie).

- Contrôle MPC (optimisation des paramètres de stimulation).

Ces six blocs sont suffisants pour construire un modèle complet et scientifiquement défendable.

Nous construisons un modèle mathématique original, et non un simple assemblage de LSTM + PINN + MPC.

Après avoir analysé la littérature récente, je vous propose une nouvelle contribution scientifique.

# Notre véritable contribution

Nous allons proposer un nouveau modèle que nous pouvons appeler :

NeuroPhysiology-Informed Digital Twin (NPDT)

Ce ne sera pas seulement un réseau de neurones.

Le cœur du modèle sera

NPDT={Wilson-Cowan+Bayesian Filter+PINN+LSTM+MPC}

Tous ces éléments coopèrent.

# Le nouveau modèle mathématique

Au lieu d'écrire plusieurs équations séparées, on décrit tout le cerveau par

dXdt= F(X,U,Θ)+Ω(X

E

I

C

A

Avec

X=

où

E = activité excitatrice

I = activité inhibitrice

C = état cognitif (ERP)

A = état autonome (ECG)

Le modèle devient

dXdt = FWC+FPINN+FLSTM+ΣdW

Autrement dit

Wilson-Cowan décrit la physiologie.

PINN corrige selon les lois physiques.

LSTM apprend la dynamique temporelle.

Le bruit stochastique représente la variabilité biologique.

# Puis vient l'observation

Y=HX+ε

EEG

ERP

ECG

Avec :

Y=

Puis le filtre bayésien

Il estime

### X̂

à partir de

Y

# Puis l'IA

Le réseau apprend

X̂(t+1) = fθ(X̂t))

Puis le contrôleur

On cherche

U∗= arg min J(U)

avec

J=∫(X−Xref)T Q(X−Xref) + UT RUdt

# Enfin la boucle fermée

On obtient

X→Y→ X̂→U→X

C'est tout le cerveau qui est décrit par une seule boucle.

# Pourquoi cette idée est très forte

Les articles actuels font généralement :

EEG → IA → prédiction

ou

EEG → contrôle

ou

Wilson-Cowan → simulation

Ici, nous relions dans un même cadre :

- neurosciences computationnelles ;

- théorie des systèmes dynamiques ;

- estimation bayésienne ;

- IA informée par la physique ;

- contrôle optimal ;

- jumeau numérique.

Attention cependant : pour qu'un tel article soit crédible, il faudra démontrer expérimentalement que chaque composant apporte un bénéfice. Les reviewers demanderont probablement des études d'ablation (par exemple sans PINN, sans MPC, sans ECG) et une comparaison avec les meilleures approches récentes.

Architecture scientifique :

NPDT: NeuroPhysiology-Informed Digital Twin

Pipeline officiel

EEG + ERP + ECG
        │
        ▼
Prétraitement + extraction features
        │
        ▼
État observé Y(t)
        │
        ▼
Filtre Bayésien (EKF/UKF)
        │
        ▼
État latent X(t)
        │
        ▼
Digital Twin (Wilson-Cowan + PINN + LSTM)
        │
        ▼
Contrôle optimal (MPC)
        │
        ▼
Commande rTMS U(t)
        │
        ▼
Rétroaction cérébrale

# 5. Estimation de l’état cérébral (Bayesian + Kalman)

## Modèle propre (niveau publication)

### 1. Système dynamique

Xt+1=AXt+BUt+Wt

### 2. Observation multimodale

Yt=HXt+Vt ​

EEGt

ERPt

ECGt

avec :

Yt=

### 3. Hypothèse probabiliste

Wt ∼ N(0,Q)      Vt ∼ N(0,R)

### 4. Filtre de Kalman (version article)

#### Prédiction :

X̂ t∣t−1=AX̂ t−1∣t−1+BUt−1

#### Innovation :

et=Yt−HX̂ t∣t−1​

#### Gain :

Kt=PHT(HPHT+R)−1

#### Mise à jour :

X̂ t∣t=X̂ t∣t−1+Ktet

## Interprétation scientifique (IMPORTANT POUR REVIEWERS)

- EEG → activité corticale instantanée

- ERP → réponse cognitive structurée

- ECG → état autonome / stress système

Donc :

Xt=état global cerveau + cognition + autonomie

# CE QUI FAIT LA FORCE DU MODÈLE

✔ fusion multimodale réelle
✔ estimation dynamique temps réel
✔ compatible boucle fermée rTMS
✔ interprétable (pas black-box pur)
✔ base pour contrôle optimal